# EVC GUI User Manual (v26.0.0)

Date: 2026-09-28

# 1. Purpose

EVC GUI is a Windows desktop tool for connecting to EVC units, validating communication, monitoring live RF metrics, and visualizing impedance behavior on Power Scope and Smith Chart.

# 2. Supported Products

Quantum

Tykon

Triton

Chronos 1 (SN prefix 191-195)

Chronos 2.0 (SN prefix 196 and above)

# 3. System Requirements

Windows 10/11

Python 3.11 or 3.12 (for source run)

Serial COM connection to EVC hardware

# 4. Start-Up and Connection

1. Click Refresh Ports and select the target COM port.

2. Choose baud (default 57600).

3. Click Start / Connect. The app validates the ver echo/content and auto-scans baud rates when mismatch/garbled data is detected.

4. On success, Detected Product and Detected Unit S/N are populated.

# 5. Run Workflow

Click Run to enter continuous pdat1/psum1 polling.

Use Abort to stop safely and return controls to idle state.

Probe is available in idle for command-path check (CR/LF).

# 6. Power Scope

Select traces to display and choose 10 sec or 60 sec window.

Click Y-axis to set Min Y, Max Y, and Step size.

For Quantum, Band selector supports HF/LF view toggling.

# 7. Smith Chart

Plots live impedance points from run data.

For Quantum, HF and LF impedance are both shown on the same chart (HF red, LF blue).

Contour overlay can be enabled with Plot Contour.

# 8. Contour Download (zpar)

Contour is auto-downloaded once after successful scan (real device mode).

Chronos 1 uses cap range 0..12/0..12; Chronos 2.0 uses Tykon-style range.

Quantum downloads both hf and lf zpar contour sets and overlays both bands on Smith Chart.

# 9. Logging and Files

Runtime logs: logs\evc_gui*.log and logs\python_fault.log

Phase 2 data logs are raw CSV only, rotated every 3 hours.

Timestamp format: YYYY-MM-DD HH:MM:SS.mmm

# 10. Build Executable

Run build_exe.ps1 in repository root to generate dist\EVC_GUI_v26.0.0.exe.

# 11. Troubleshooting

If Start/Connect fails, confirm COM port ownership and cable/device power.

If data appears garbled, verify that auto-baud scan completed and product/SN were detected.

If contour fails, check status log for zpar command responses and device support for requested band.